LIFE OF A PACKET

0️⃣ You type facebook.com and press Enter

At this point, nothing has gone on the wire yet.

The browser first answers:

- Is this HTTP or HTTPS? → modern browsers assume HTTPS

- Do I already know the IP?

- Do I have an open connection I can reuse?

1️⃣ URL parsing (browser logic)

Browser breaks the URL into parts:

| Part | Value |
| --- | --- |
| Scheme | https |
| Hostname | facebook.com |
| Port | 443 (default for HTTPS) |
| Path | / |

If HTTPS → browser knows:

- TLS handshake is required

- TCP must be established first

2️⃣ Browser cache check (cheap wins first)

Before touching the network:

- DNS cache (browser-level)

- OS DNS cache

- Connection reuse (HTTP/2 / HTTP/3)

If IP is cached → skip DNS entirely
(huge performance win)

👉 As a network engineer, this is why:

nslookup facebook.com

might show a delay but browser loads instantly.

3️⃣ DNS resolution (this is where networking starts)

If IP not cached:

3.1 Stub resolver (your OS)

Your OS asks:

“Who is my DNS server?”

Usually:

- ISP DNS

- Corporate DNS

- Public DNS (8.8.8.8, 1.1.1.1)

3.2 Recursive DNS query (step-by-step)

Your DNS server does not magically know Facebook.

It walks the DNS tree:

Root DNS (.)

↓

.com TLD server

↓

facebook.com authoritative server

Each step returns:

- NS records

- A / AAAA records

- TTL values

Eventually:

facebook.com → 157.240.xxx.xxx

✔ Multiple IPs returned (load balancing, geo-DNS)
✔ TTL controls caching behavior

👉 Engineer mindset:

- Slow site? Check DNS latency

- Wrong site? DNS poisoning / hijack

4️⃣ ARP (local network reality check)

Now your machine knows Facebook’s IP — but…

“How do I actually send packets?”

4.1 Is the destination local?

- Compare destination IP with subnet mask

- Result: ❌ not local

So traffic must go to:
➡ Default Gateway

4.2 ARP request

If MAC of gateway unknown:

Who has 192.168.1.1? Tell 192.168.1.10

Gateway replies with MAC address.

Now your PC can build an Ethernet frame.

5️⃣ TCP 3-way handshake (transport layer)

Facebook uses TCP (still, even with HTTP/3 rising).

5.1 SYN

SRC: random high port (e.g. 52134)

DST: 443

Flags: SYN

5.2 SYN-ACK

Facebook server replies:

SYN + ACK

5.3 ACK

Client acknowledges.

🎉 TCP session established

👉 Things negotiated here:

- MSS

- Window scaling

- SACK

👉 Troubleshooting clues:

- SYN sent, no reply → firewall / routing issue

- SYN-ACK seen, ACK missing → client issue

6️⃣ TLS handshake (security layer)

Now encryption starts.

6.1 ClientHello

Browser sends:

- TLS version

- Cipher suites

- SNI = facebook.com (important!)

- ALPN (HTTP/1.1, HTTP/2, HTTP/3)

6.2 ServerHello

Facebook replies:

- Selected cipher

- Certificate chain

- Public key

6.3 Certificate validation

Browser checks:

- Signed by trusted CA?

- Domain matches?

- Expired?

- Revoked?

If any check fails → big red warning 😬

6.4 Key exchange

- Session keys generated

- Symmetric encryption starts

From now on:
🔒 Payload is encrypted
(you only see metadata on the wire)

7️⃣ HTTP request (finally!)

Now the browser sends:

GET / HTTP/2

Host: facebook.com

User-Agent: Chrome/144

Cookies: session, tracking

Over:

- TLS

- TCP

- IP

- Ethernet/Wi-Fi

8️⃣ Facebook’s backend magic

Inside Facebook’s network:

- Load balancer (L4/L7)

- Geo-routing

- CDN (images, JS, CSS)

- Cache lookup

- App servers

- Databases

Your request may hit:

- Singapore POP

- Or nearest APAC DC

👉 IP you connected to ≠ actual app server

9️⃣ HTTP response

Server replies:

- Status code: 200 OK

- HTML content

- Headers:

  - Cache-Control

  - Content-Security-Policy

  - Set-Cookie

Browser receives initial HTML only.

🔟 Page explosion (most people forget this)

HTML is parsed and triggers dozens / hundreds of new requests:

- CSS

- JS

- Images

- Fonts

- Ads

- Tracking pixels

- API calls

Modern browsers:

- Use HTTP/2 multiplexing

- Reuse same TCP connection

- Parallelize aggressively

This is where performance lives or dies.

1️⃣1️⃣ Rendering (client-side)

Browser pipeline:

- Build DOM

- Build CSSOM

- Create render tree

- Layout

- Paint

- Composite

JS can:

- Block rendering

- Trigger reflows

- Make async API calls

1️⃣2️⃣ Connection persistence

Connection stays open:

- Keep-Alive

- Reused for future requests

- Closed after idle timeout